ResQNet

Phase 1: Requirements and Network Design

Intelligent Emergency Response and Hospital Network Using Dynamic Routing  |  DCCN Lab (CMP-303)  |  Week 1 to 2

# 1. What is Phase 1?

Phase 1 is the design and planning stage. The goal is to freeze the complete network topology, device inventory, IPv4 addressing plan, and dynamic routing scheme on paper before touching Cisco Packet Tracer.

Phase 1 is done when: the topology is finalized, every department LAN is calculated as a /26 subnet (62 hosts), every WAN link is calculated as a /30 subnet (2 hosts), the OSPF Area 0 routing hierarchy is designed with backup costs, and the access security policy is verified.

# 2. What you do in Phase 1

| Task | What it means |
| --- | --- |
| Select hardware models | Specify Cisco 3560 Core Switch, 2911 Routers, 2960 Switches, and Server-PT. |
| Design IPv4 LAN addressing | Subnet 10.N.0.0/26 for hospitals, 10.10.0.0/26 for control room, and 10.20.0.0/26 for ambulances. |
| Assign static server IPs | Allocate 10.10.0.66 for Web/DNS server and 10.10.0.67 for centralized DHCP relay server. |
| Design point-to-point WAN links | Assign 172.16.0.0/30 subnets for primary and backup links. |
| Architect OSPF dynamic routing | Define Area 0, unique router IDs (1.1.1.1 to 6.6.6.6), cost 50 on backup links, and passive LAN ports. |
| Formulate ACL security policies | Draft extended ACL rules ensuring department isolation and DHCP pass-through. |

# 3. Hardware Inventory and Roles

| Device Identifier | Model Suggestion | Role in Network | Key Interfaces & Connectivity |
| --- | --- | --- | --- |
| R-CORE | Cisco Catalyst 3560 | Backbone Layer 3 switch | Gig0/1 to CTRL, Gig0/2 to H1, Gig0/3 to H2, Gig0/4 to H3, Gig0/5 to AMB |
| R-CTRL | Cisco 2911 Router | Control room & server farm gateway | Gig0/0: Trunk to SW-CTRL, Gig0/1: to CORE, Gig0/2: Backup to H3 |
| R-H1 | Cisco 2911 Router | Hospital 1 (North) gateway | Gig0/0: Trunk to SW-H1, Gig0/1: to CORE, Gig0/2: Backup to H2 |
| R-H2 | Cisco 2911 Router | Hospital 2 (Central) gateway | Gig0/0: Trunk to SW-H2, Gig0/1: to CORE, Gig0/2: Backup to H1 |
| R-H3 | Cisco 2911 Router | Hospital 3 (South) gateway | Gig0/0: Trunk to SW-H3, Gig0/1: to CORE, Gig0/2: Backup to CTRL |
| R-AMB | Cisco 2911 Router | Ambulance centres 1 & 2 gateway | Gig0/0: Trunk to SW-AMB, Gig0/1: to CORE |
| SW-CTRL | Cisco Catalyst 2960 | Access layer for control room | Fa0/24: Trunk to R-CTRL, Fa0/1: Staff, Fa0/2-3: Servers |
| SW-H1..H3 | Cisco Catalyst 2960 | Hospital access switches | Fa0/24: Trunk to router, Fa0/1-23: Access ports for VLANs |
| SW-AMB | Cisco Catalyst 2960 | Ambulance access switch | Fa0/24: Trunk to R-AMB, Fa0/1-20: Access ports for VLANs |
| SRV-PORTAL | Server-PT | Emergency Web & DNS server | Static IP 10.10.0.66/26, portal.resqnet.local |
| SRV-DHCP | Server-PT | Centralized DHCP server | Static IP 10.10.0.67/26, pools for all site VLANs |

# 4. Full IPv4 Addressing Plan

Addressing Logic: Every site LAN is sized as a /26 subnet (mask 255.255.255.192), accommodating 62 usable hosts per department. Every WAN router link is a /30 subnet (mask 255.255.255.252), accommodating exactly 2 usable host addresses with no wasted space.

## 4.1 Site LAN Subnets (Hospitals, Control Room, Ambulances)

| Site | VLAN | Department / Purpose | Subnet CIDR | Subnet Mask | Default Gateway | Usable Range |
| --- | --- | --- | --- | --- | --- | --- |
| Hospital N (1-3) | 110 | Administration | 10.N.0.0/26 | 255.255.255.192 | 10.N.0.1 | 10.N.0.2 - 10.N.0.62 |
| Hospital N (1-3) | 120 | Doctors | 10.N.0.64/26 | 255.255.255.192 | 10.N.0.65 | 10.N.0.66 - 10.N.0.126 |
| Hospital N (1-3) | 130 | Nurses | 10.N.0.128/26 | 255.255.255.192 | 10.N.0.129 | 10.N.0.130 - 10.N.0.190 |
| Hospital N (1-3) | 140 | Emergency | 10.N.0.192/26 | 255.255.255.192 | 10.N.0.193 | 10.N.0.194 - 10.N.0.254 |
| Control Room | 10 | Control Staff | 10.10.0.0/26 | 255.255.255.192 | 10.10.0.1 | 10.10.0.2 - 10.10.0.62 |
| Control Room | 20 | Servers (Web/DNS/DHCP) | 10.10.0.64/26 | 255.255.255.192 | 10.10.0.65 | 10.10.0.68 - 10.10.0.126 |
| Ambulance 1 | 10 | Dispatch PCs | 10.20.0.0/26 | 255.255.255.192 | 10.20.0.1 | 10.20.0.2 - 10.20.0.62 |
| Ambulance 2 | 20 | Dispatch PCs | 10.20.0.64/26 | 255.255.255.192 | 10.20.0.65 | 10.20.0.66 - 10.20.0.126 |

## 4.2 Dedicated Server IP Assignments

| Server Name | Service Function | Assigned IP | Subnet Mask | Default Gateway | VLAN |
| --- | --- | --- | --- | --- | --- |
| portal.resqnet.local | Emergency Portal (HTTP:80) & DNS (53) | 10.10.0.66 | 255.255.255.192 | 10.10.0.65 | VLAN 20 (SW-CTRL Fa0/2) |
| dhcp.resqnet.local | Central DHCP Server (UDP:67) | 10.10.0.67 | 255.255.255.192 | 10.10.0.65 | VLAN 20 (SW-CTRL Fa0/3) |

## 4.3 Point-to-Point WAN Links (/30)

| Link Segment | Subnet CIDR | Side A Port & IP | Side B Port & IP | Link Type | OSPF Cost |
| --- | --- | --- | --- | --- | --- |
| CORE to CTRL | 172.16.0.0/30 | R-CORE Gig0/1 (.1) | R-CTRL Gig0/1 (.2) | Primary | 1 (Default) |
| CORE to H1 | 172.16.0.4/30 | R-CORE Gig0/2 (.5) | R-H1 Gig0/1 (.6) | Primary | 1 (Default) |
| CORE to H2 | 172.16.0.8/30 | R-CORE Gig0/3 (.9) | R-H2 Gig0/1 (.10) | Primary | 1 (Default) |
| CORE to H3 | 172.16.0.12/30 | R-CORE Gig0/4 (.13) | R-H3 Gig0/1 (.14) | Primary | 1 (Default) |
| CORE to AMB | 172.16.0.16/30 | R-CORE Gig0/5 (.17) | R-AMB Gig0/1 (.18) | Primary | 1 (Default) |
| H1 to H2 | 172.16.0.20/30 | R-H1 Gig0/2 (.21) | R-H2 Gig0/2 (.22) | Backup | 50 (Failover) |
| H3 to CTRL | 172.16.0.24/30 | R-H3 Gig0/2 (.25) | R-CTRL Gig0/2 (.26) | Backup | 50 (Failover) |

# 5. Routing Architecture: OSPF Area 0

1. Single Backbone Area: All routers participate in single Area 0 (OSPF process 1).
2. Deterministic Router IDs:
   - R-CORE: 1.1.1.1
   - R-CTRL: 2.2.2.2
   - R-H1: 3.3.3.3
   - R-H2: 4.4.4.4
   - R-H3: 5.5.5.5
   - R-AMB: 6.6.6.6
3. High-Cost Backup Paths: The backup links (H1-H2 and H3-CTRL) are assigned 'ip ospf cost 50'. Under normal operation, OSPF chooses the lowest-cost path (primary links cost 1). Only upon link failure does Dijkstra's SPF algorithm redirect traffic over the backup.
4. Network Point-to-Point: All WAN links run 'ip ospf network point-to-point', bypassing DR/BDR election overhead and speeding convergence.
5. Passive Interfaces: 'passive-interface default' is applied to prevent OSPF Hello packet leaks onto user VLANs.

# 6. Access Control Policy (Security Matrix)

| Source | Destination | Protocol / Port | Rule | Security Objective |
| --- | --- | --- | --- | --- |
| Every VLAN | SRV-PORTAL (10.10.0.66) | HTTP (80), DNS (53), ICMP | Permit | Allow all staff and ambulances to reach the emergency portal. |
| Every VLAN | SRV-DHCP (10.10.0.67) | UDP BootPS (67) | Permit | Allow DHCP relay requests to reach central server. |
| Nurses (VLAN 130) | Admin (VLAN 110) | All IP | Deny | Protect administrative financial/personnel systems from ward terminals. |
| Admin (VLAN 110) | Doctors (VLAN 120) | All IP | Deny | Isolate medical diagnosis systems from general administrative staff. |
| Emergency (VLAN 140) | Control Staff (10.10.0.0/26) | All IP | Permit | High-priority direct line between ER and Control Room. |
| Ambulance Centres | Hospital Admin / Nurses | All IP | Deny | Ambulances cannot access clinical patient ward records. |
| Any Host PC | Routers (VTY) | SSH / Telnet | Deny | Management access restricted strictly to Control Staff VLAN 10. |

# 7. Sign-Off and Readiness Checklist

| Deliverable Item | Criteria | Verification Status |
| --- | --- | --- |
| Topology Plan | All 6 routers, 5 site switches, and 2 servers mapped to ports | COMPLETED & FROZEN |
| LAN Addressing | All 16 LANs assigned non-overlapping /26 blocks (62 hosts each) | COMPLETED & FROZEN |
| WAN Addressing | All 7 WAN links assigned distinct /30 blocks (2 hosts each) | COMPLETED & FROZEN |
| Server Allocation | Fixed IPs assigned for portal (10.10.0.66) and DHCP (10.10.0.67) | COMPLETED & FROZEN |
| OSPF Routing Spec | Area 0, Router IDs 1.1.1.1 to 6.6.6.6, cost 50 backup links defined | COMPLETED & FROZEN |
| ACL Policy Spec | Department separation matrix and DHCP permit rules verified | COMPLETED & FROZEN |
| GitHub Repository | docs/PHASE1.md and docs/addressing_plan.csv committed and merged | COMPLETED & FROZEN |