ResQNet

Phase 0: Team, Tools and Setup

Intelligent Emergency Response and Hospital Network Using Dynamic Routing  |  DCCN Lab (CMP-303)  |  Week 1

# 1. What is Phase 0?

Phase 0 is the getting-ready week. Nothing is built yet. The goal is simple:

Everyone on the team can open the same project folder and run the same tools.

Phase 0 is done when: all roles are assigned, every team member has installed the tools and passed the check, the project folder is shared, and the file naming rules are agreed.

# 2. What you do in Phase 0

| Task | What it means |
| --- | --- |
| Assign roles | Decide who does what (section 6) and write names in docs/TEAM.md. |
| Install tools | Install the software on every laptop (section 4). |
| Check the setup | Run the check script and confirm no FAIL lines (section 5). |
| Share the folder | Give every member a copy of the project folder (section 3). |
| Agree the rules | File naming for Packet Tracer and how to save router configs (section 3). |

Packet Tracer in Phase 0: you only install it and open it once to check it works. You do not build any network in it yet. That starts in Phase 2.

# 3. The project folder and team rules

## 3.1 Folder structure

| Folder | What goes in it |
| --- | --- |
| pkt/ | Packet Tracer files: resqnet_v1.pkt, resqnet_v2.pkt, ... (empty until Phase 2) |
| configs/ | Router and switch configs saved as text after each phase |
| scripts/ | check_env.py now; test and measurement scripts later |
| dashboard/ | The web dashboard (built in Phase 5) |
| captures/ | Wireshark captures and test screenshots |
| docs/ | PHASE0.md, TEAM.md, and later the report and demo script |

## 3.2 Sharing the folder

- Extract resqnet_phase0_plain.zip to a folder on one laptop, for example C:\Users\91839\OneDrive\Desktop\resqnet

- Put a copy in a shared place (Google Drive, OneDrive or a pen drive).

- Every member copies it to their own laptop.

- Copy the whole folder to a second place at the end of every work session as a backup.

## 3.3 The Packet Tracer file rule

Packet Tracer (.pkt) files cannot be merged, so two people's edits cannot be combined. Follow these rules:

- Name files resqnet_v1.pkt, resqnet_v2.pkt, and so on.

- Never overwrite a working version. Open the latest, then File > Save As with the next number.

- Only one person edits at a time. Announce it in the team chat ("I have v4").

- Log each version in pkt/README.md.

## 3.4 The config-saving rule

After every phase, on every router and switch, type:

copy running-config startup-config

show running-config

Then paste the output into configs/<phase>/<device>.txt, for example configs/phase2/R-H1.txt. These text files let you rebuild the network if the .pkt file is lost or corrupted.

# 4. Tools to install

| Tool | Used for | Where to get it | Needed? |
| --- | --- | --- | --- |
| Cisco Packet Tracer | Building the network (Phase 2 onward) | netacad.com (free Networking Academy account) | Yes |
| Wireshark | Looking at network packets | wireshark.org | Yes |
| Python 3 | Test scripts and the dashboard | python.org | Yes |
| VS Code | Editing scripts and documents | code.visualstudio.com | Recommended |
| draw.io | Topology and flow diagrams | diagrams.net | Recommended |
| Ubuntu VM or WSL2 | Running the measured twin network | Microsoft Store or any VM tool | Optional |
| Mininet, FRRouting | Measured twin network (Layer B) | sudo apt install mininet frr (inside Ubuntu) | Optional |

Tip: sign up for the Networking Academy account on day one. Approval and download can take a day or more, and Packet Tracer cannot be downloaded without it.

Suggested laptop: 8 GB RAM or more, Windows or Linux.

## Python packages

Open PowerShell in the project folder and run:

py -m pip install -r requirements.txt

# 5. Check that your setup works

From the project folder, run:

py scripts\check_env.py

(On Linux or macOS use python3 scripts/check_env.py.)

| Result | Meaning |
| --- | --- |
| PASS | Found and working. |
| WARN | An optional tool is missing, or a program the script cannot detect (Packet Tracer). |
| FAIL | A required item is missing. Install it and run the script again. |

The script can only look for Packet Tracer in common install folders, so a WARN for it is normal. Check it yourself: open Packet Tracer, sign in with your Networking Academy account, and open a blank project. The Mininet and FRRouting warnings can be ignored unless you try the optional measured twin.

# 6. Team roles

Write names and roll numbers in docs/TEAM.md. If the team is smaller than four, merge roles.

| Role | Owns | Main phases |
| --- | --- | --- |
| Network lead | Topology, addressing, routers, OSPF | 1, 2, 4 |
| Services and security lead | DHCP, DNS, HTTP portal, VLANs, ACLs | 2, 3 |
| Dashboard developer | Web dashboard, test scripts, twin network | 5 |
| Testing and documentation lead | Experiments, Wireshark, synopsis, report, demo | 4, 6 |

- Team of three: the Network lead also runs the experiments, and the Dashboard developer writes the documentation.

- Team of two: pair Network with Services, and Dashboard with Testing.

# 7. Sign-off checklist

Phase 0 is complete when every box is ticked.

☐  All roles assigned in docs/TEAM.md, and the lab submission date recorded

☐  Project folder copied to every member's laptop, with a shared copy agreed

☐  Packet Tracer installed and opening a blank project on every laptop

☐  Wireshark and Python 3 installed on every laptop

☐  scripts/check_env.py shows no FAIL on every laptop

☐  Packet Tracer naming rule (resqnet_vN.pkt) and one-editor rule agreed

☐  Config-saving rule agreed and understood

☐  Decision made: try the measured twin (Layer B), or start with Packet Tracer only

| Member | check_env.py passed | Packet Tracer opens | Date |
| --- | --- | --- | --- |

# 8. Risks in this phase

| Risk | What to do |
| --- | --- |
| Two people save over the same .pkt file | One editor at a time, numbered file names, and a version log in pkt/README.md. |
| The .pkt file is lost or corrupted | The saved text configs in configs/ rebuild every device. |
| Packet Tracer access is delayed | Start the Networking Academy sign-up on day one. |
| The twin network takes too much time later | Decide now: Packet Tracer first, and add the twin as an upgrade. |
| A Windows laptop cannot run Mininet | Use WSL2 or an Ubuntu VM, or skip the twin. |

# 9. What comes next

Phase 1 (Requirements and design) fixes the topology and the addressing plan on paper before anything is built in Packet Tracer. The addressing in the implementation plan is already complete and checked, so Phase 1 is mainly a review and approval step. Bring the team sheet, the signed-off checklist from section 7, and a draw.io topology diagram based on Figure 1 of the synopsis.